Ex.No.4:  OFDMA Resource Block Allocation

AIM:

To simulate and analyze OFDMA (Orthogonal Frequency Division Multiple Access) resource block allocation and evaluate system performance in terms of resource utilization and user throughput using MATLAB.

# Objectives:

1. To determine the Number of Resource Blocks (RBs) allocated in an OFDMA system based on available bandwidth and system configuration.

2.  To evaluate the Resource Block Utilization (%) by analyzing how efficiently available RBs are assigned to active users.

3.  To calculate the User Throughput (Mbps) based on allocated resource blocks and modulation efficiency in the OFDMA system.

Objective 1 Matlab Code and Output:

clc; clear; close all;

% System parameters

bandwidth = 10e6;              % 10 MHz

rb_size = 180e3;               % 180 kHz per RB

% Total Resource Blocks

total_RB = floor(bandwidth / rb_size);

% Users

users = 1:6;

% RB allocation (simple equal + remainder distribution) base = floor(total_RB / length(users));

alloc = base * ones(1,length(users)); alloc(1:mod(total_RB,length(users))) = alloc(1:mod(total_RB,length(users))) + 1;

utilization = (alloc / total_RB) * 100;

disp('Total Resource Blocks:') disp(total_RB)

disp('RB Allocation per User:') disp(alloc)

figure

% -------- Graph 1: RB Allocation per User -------- subplot(2,1,1)

stem(users, alloc, 'filled', 'LineWidth', 2) title('OFDMA Resource Block Allocation per User')

xlabel('User Index') ylabel('Number of Resource Blocks')

grid on

% -------- Graph 2: RB Utilization -------- subplot(2,1,2) plot(users, utilization, '-o', 'LineWidth', 2) title('OFDMA Resource Block Utilization (%)')

xlabel('User Index') ylabel('Utilization (%)') grid on

Objective 2 Matlab Code and Output:

clc; clear; close all;

% Total system resource blocks total_RB = 50;

% Number of active users

users = 1:5;

% RB allocation to users (example scenario)

alloc = [12 10 9 8 11];

% Resource Block Utilization (%)

util = (alloc / total_RB) * 100;

disp('Resource Block Utilization (%) per User:') disp(util')

figure

% -------- Graph 1: RB Allocation -------- subplot(2,1,1)

plot(users, alloc, '-o', 'LineWidth', 2) title('OFDMA Resource Block Allocation per User') xlabel('User Index') ylabel('Number of Resource Blocks') grid on

% -------- Graph 2: Utilization (%) -------- subplot(2,1,2)

plot(users, util, '-s', 'LineWidth', 2) title('OFDMA Resource Block Utilization (%)')

xlabel('User Index') ylabel('Utilization (%)') grid on

Objective 3 Matlab Code and Output:

clc; clear; close all;

% System parameters

rb_bw = 180e3;                 % Bandwidth per RB (180 kHz) users = 1:5;

% Allocated Resource Blocks per user alloc_RB = [12 10 9 8 11];

% Modulation efficiency (bits/symbol) mod_eff = [2 4 6 4 2];  % QPSK, 16QAM, 64QAM etc.

% Throughput calculation (Mbps)

throughput = (alloc_RB .* rb_bw .* mod_eff) / 1e6;

disp('User Throughput (Mbps):') disp(throughput')

figure

% -------- Graph 1: Throughput per User -------- subplot(2,1,1)

plot(users, throughput, '-o', 'LineWidth', 2) title('OFDMA User Throughput') xlabel('User Index') ylabel('Throughput (Mbps)')

grid on

% -------- Graph 2: RB vs Throughput -------- subplot(2,1,2) plot(alloc_RB, throughput, '-s', 'LineWidth', 2) title('Resource Blocks vs Throughput Relationship') xlabel('Allocated Resource Blocks') ylabel('Throughput (Mbps)')

grid on