Simulation of FDMA in MATHLAB  date

Aim:

To simulate a Frequency Division Multiple Access (FDMA) system using MATLAB  and analyze its communication performance in terms of channel bandwidth, data rate, and  signal-to-noise ratio (SNR)

Objective 1

clc;

clear;

close all;

% User and Slot Configuration

Users = 1:8;             % Total number of users

TotalSlots = 5;          % Available TDMA slots

% Initialize slot allocation

Alloc = zeros(1, length(Users));

% Allocate slots to the first users

Alloc(1:TotalSlots) = 1;

% Identify blocked users

Blocked = 1 - Alloc;

% Display User-wise Slot Allocation

disp('User Wise Slot Allocation');

AllocationTable = table(Users', Alloc', ...

'VariableNames', {'User', 'Status'});

disp(AllocationTable);

% Create Figure

figure;

% Graph 1: Allocation per User

subplot(2,1,1);

stem(Users, Alloc, 'filled', 'LineWidth', 2);

title('TDMA Slot Allocation per User');

xlabel('User Index');

ylabel('Slot Status (1 = Allocated, 0 = Blocked)');

ylim([-0.2 1.2]);

xticks(Users);

grid on;

% Graph 2: System View

subplot(2,1,2);

plot(Users, Alloc, '-o', 'LineWidth', 2);

hold on;

plot(Users, Blocked, '-s', 'LineWidth', 2);

title('TDMA Slot Utilization Analysis');

xlabel('User Index');

ylabel('Status');

ylim([-0.2 1.2]);

xticks(Users);

legend('Allocated', 'Blocked', 'Location', 'best');

grid on;

hold off;

Objective 2

clc;

clear;

close all;

BW = 200;          % Total Channel Bandwidth (kHz)

Users = 3;         % Number of Users

M = 2;             % BPSK Modulation

% Bandwidth allocated to each user

UserBW = BW / Users;

% Data rate for each user

DataRate = UserBW * log2(M);

% Data rate for all users

Rate = DataRate * ones(1, Users);

% Display results

disp('FDMA User Data Rate (kbps)');

disp([(1:Users)' Rate']);

% Plot bar graph

figure;

bar(1:Users, Rate);

xlabel('Users');

ylabel('Data Rate (kbps)');

title('FDMA Data Rate for Each User');

grid on;

Objective 3

clc;

clear;

close all;

Ps = 1;                         % Signal Power (W)

Pn = [0.1 0.05 0.01];           % Noise Power for each user (W)

Users = 1:3;                    % Number of Users

% Calculate SNR in dB

SNR = 10 * log10(Ps ./ Pn);

% Display results

disp('User    Noise Power (W)    SNR (dB)');

disp([Users' Pn' SNR']);

% Plot SNR

figure;

bar(Users, SNR);

xlabel('Users');

ylabel('SNR (dB)');

title('Signal-to-Noise Ratio of FDMA Users');

grid on;

Result:

1. The FDMA system was successfully simulated by allocating separate frequency channels  to three users, and the channel bandwidth assigned to each user was determined. 2. The data rate achieved by each user was successfully calculated based on the allocated  channel bandwidth, demonstrating equal transmission capacity for all users. 3. The Signal-to-Noise Ratio (SNR) for different users was successfully evaluated,  showing that higher SNR values provide better communication quality and reliable data  transmission.